Git: https://github.com/Chetan-379/GEANT-4.git

Color Calorimetry

Slides for verifying: link

(A) Instructions for Angle reconstruction in a pixelated setup:

Note:

- right now the angle reconstruction is done using the true Edep info, not simulating the optical photons there.

- You will have to write by your own the analyzer script to scattering angle reconstruction with optical photons (spread the chosen energy ranges according to the energy resolution).

For the details of the approach followed for angle reconstruction: slide: 16

To Run the simulation:

clone the git repository:

git clone --branch CCal_OpOFF_angle_reco --depth 1 https://github.com/Chetan-379/GEANT-4.git

(a) simulate with CCal setup:

- cd GEANT-4/CCAL_MultiPixel

- cp CCal_construction construction.cc

- mkdir build

- cd build

- cmake .. && make

- ./CCal run.mac

- mkdir -p ../Analyze/src_root_files

- cp test.root ../Analyze/src_root_files/OpOFF_rep_CCal_100kEvts.root

(b) simulate without CCal setup:

- Modify the ‘construction.cc’ file (out of the build folder):

  - cd .. (outside the build folder)

  - cp noCCal_constuction construction.cc

  - Repeat from step 4-6 of CCal case

  - cp test.root ../Analyze/src_root_files/OpOFF_noCCal_100kEvts.root

To do the analysis:

- cd ../Analyze && make

- for CCal case:

./analyzeLightBSM CCal_inputfiles.txt OpOFF_CCal_100kEvts_out.root GAGG_LYSO

- for no CCal case:

./analyzeLightBSM noCCal_inputfiles.txt OpOFF_noCCal_100kEvts_out.root GAGG

For verifying:
A. for CCal case

output root file name: OpOFF_CCal_100kEvts_out.root

Relevant histograms:

- reco theta w/o condition on interactions: scat_theta_Ana (check slide 18)

- truth theta w/o condition on interactions: scat_theta_Sim

- Reco vs truth w/o …: theta_SimVsAna

- Difference in reco and sim w/o….: dTheta (slide 22)

- For number of interactions condition applied check the similar distribution with “_nInt” at the histogram name (slide: 19, 23).

B. for noCCal case:

output root file name: OpOFF_noCCal_100kEvts_out.root GAGG

- Relevant histograms have similar names as the CCal case but with “_noCCal” at the end of the histogram name.

- For without condition on interactions: check slide 20,22

- For condition on interactions: check slide 21,23

*************************************************************************************************************

(B) instructions for simulating with optical photons and applying quantum efficiency in CCal with a single crystal:

- Clone the git repository:

git clone --branch Applied_QE --depth 1 https://github.com/Chetan-379/GEANT-4.git

- Run the following commands one by one to run the simulation:

- cd GEANT-4/CCAL_MultiPixel

- mkdir build && cd build

- cmake .. && make

- ./CCal_OpON (to run with visualization)

- ./CCal_OpON run.mac (to run without visualization. Change the number of events in the run.mac file to 100 to match the results with the slides.)

- For verification:

In the root file ‘test_vis.root’ match the following histograms with slide 8:

- Op_lambda    : distribution of optical photon wavelength reaching to rear surface

- Op_QE_lambda : ------------ After applying QE

- Op_energy: ------------------- Op energy distribution

- Op_energy: -----------------------------------------------After applying QE

- To run the Analyzer (simulate for 10000 events, which will take longer)

- mkdir -p ../Analyze/src_root_files

- cp test_vis.root ../Analyze/src_root_files/correct_QE_10kEvts.root

- cd ../Analyze/ && make

- ./analyzeLightBSM inputfiles.txt OpON_CCal_100Evts_out.root GAGG_LYSO

Match the following distributions with slide slide 9

- n_Op_GAGG: no. of optical photons produced in GAGG

- n_Op_GAGG_QE :after applying QE

Note: numbers will match exactly when made the plots using the plotting script: ‘overlay.C’ in the present in the ‘Analyze directory’.

*************************************************************************************************************

Some general comments:

- If you want to store any new branch in the output root file of simulation program, add that into run.cc file.

  - Modify the NtupleVariables.h accordingly to run the analyze the variables of that branch.

- For any changes modify the ‘AnalyzeLightBSM.cc’. To define more histograms write in ‘AnalyzeLightBSM.h’ file.

- For running the analyzer, the second argument (a .txt file) contains the name of the root file to be analyzed.

- While running a simulation program there you can give the desired output file name as an argument also for e.g. ./CCal run.mac CCal_100k_evts.root

( in the current machinery, only possible with running with mac file)

- If you want switch on or off the optical photons in the material uncomment or comment respectively following lines in the ‘construction.cc’:

gagg->SetMaterialPropertiesTable(G_MPT);

lyso->SetMaterialPropertiesTable(L_MPT);

- You might have to change the construction composition of LYSO.

========================================================================

Polarization study in multipixelated GAGG setup

Note: the scripts in this version are not fully correct !!

- The codes can be found in the tag named ‘working_dphi_truth’.

- There the directory ‘OpOFF_MultPixGAGG’ contains the scripts to simulate and analyze the things without optical photons and OpON_MultiPixGAGG with optical photons.

- For simulating follow the similar processes as the CCal one, keep in mind to change the paths and filenames accordingly.

- Running the analyzer is also similar. Use the file ‘get_dphi_RmBkg.C’ to divide any two histograms of two root file. (needed for getting the corrected dPhi distribution)

- Relevant histograms:

(A) without optical photons:

  - diff_Phi_truth_inc (all scattering angles)

  - diff_Phi_truth for theta 72 to 90 degrees

(B) with optical photons:

  - dPhi_reco_inc

  - dPhi_reco_70_90

To simulate with Optical photons run the simulation on condor:

  - Relevant tag: ‘G4_condor’

- The bash script ‘condor.sh’ is the main executable which is executed at any node.

  - Give the correct paths according to your system, where all the files necessary for simulation are present.

  - Comment out the line

“export X509_USER_PROXY=/afs/cern.ch/user/c/cagrawal/x509up_u165745”

if needed.

  - For starting you can comment out the commands starting with ‘mv’ there. These are just to not fill the afs area with log files and root files.

- The file ‘proto_condor_submit’ is the main template file for any job.

  - It contains the different paths and instructions for a job to be run.

  - It contains the some place holders such as ‘JobID’, ‘outfile’, ‘execute’, ‘case’.

  - Modify all the paths according to your system except those placeholders.

- The python file ‘condor.py’ replaces those placeholders present with the relevant strings.

  - There give the number of jobs you want to run in the variable ‘max_count’.

  - Modify the paths there and make the relevant directories in the relevant areas.

- The python file ‘submit.py’ submits all the jobs one by one.

  - Specify the number of jobs you want to run in max_count variable there.

Typical sequence for running the simulation on condor

- Clone the git repository in your afs area on LXPLUS:

git clone --branch G4_condor --depth 1 https://github.com/Chetan-379/GEANT-4.git

- cd GEANT-4/Condor_OpON_MultPixGAGG/condor

- Do the relevant modifications in the paths and make the relevant directories in the relevant places.

- python3 condor.py

- Python3 submit.py

Some general Comments:

- Simulations with optical photons are generated with different random seed each time. Therefore numbers won’t match exactly. However the raw root files (persent in the eos area) contains a branch having the random seed which was used to create that root file.

- For implementing the CCal in multipixel GAGG setup, I recommend to use the Both simulation and Analysis machinery of CCal step and do the modifications there. Because here both simulation and analysis machinery are very inefficient computationally.

- You can run any stand alone G4 simulation program on LXPLUS by setting using the environment for G4 using the command:

source /cvmfs/sft.cern.ch/lcg/views/LCG_107/x86_64-el9-gcc14-opt/setup.sh

And then the usual way of running the simulation inside the ‘build’ folder.

==================================================================

Energy resolution and efficiency studies

Steps to generate the Tree and Analyze the tree to calculate Resolution, Efficiency and make the plots for PbWO4 10cm thickness:

- clone the git repository:

git clone --branch LYSO_Resol_Eff --depth 1 https://github.com/Chetan-379/GEANT-4.git

- Go to the working directory: cd GEANT-4/diff_mat_scint

- Generate the tree for 10 cm PbWO4: (for other thickness modify the line "G4double rad_dz = 10 * cm" in "PbWO4_construction construction.cc" file.)

  - cp PbWO4_construction construction.cc (Use BGO instead of PbWO4 if you want to generate for BGO)

  - mkdir build && mkdir -p Analyze/src_root_files && mkdir -p Analyze/out_root_files && cd build (replace PbWO4 with BGO if want for BGO)

  - cmake ..

  - cp ../runJob.sh ./ && ./runJob.sh

- Analyze the tree:

  - cd ../Analyze/ && ./make_hists.sh

  - ./make_hists.sh

- Do the fitting and store the plots (resolution and efficiency calculation):

  - ./make_dir.sh

  - root -b ‘FitFunc.C(“./”)’

- Plots can be found in the ‘plots’ directory

- To make the plots of the physical variables of Scintillation photons use plot_all_hists.C.

- To overlay the data from different txt files use: Overlay_txt.C.

- To plot data from a single txt file use: plot_txt.C

Some general comments:

- The latest version (at the time of thesis writing) can be found in the git tag (though not any major edits):

https://github.com/Chetan-379/GEANT-4/tree/res_eff_before_hackathon

- Find a cleaner and automated version of fitting and plotting script in the git tag:

https://github.com/Chetan-379/GEANT-4/tree/overlay_plotting_scripts

- References for different scintillation parameters: ScintillatorParameterValues

- Values of energy resolution and efficiency: Resol_Eff_Tables

========================================================================

Polarization studies in a JPET like Geometry

- To run the simulation:

- clone the git repository:

git clone --branch Cleaned_Till_IMPETUS2 --depth https://github.com/Chetan-379/GEANT-4.git

- cd GEANT-4/Geometry

- mkdir build

- cmake ..

- ./test

- To run the Analyzer:

- cd Analyze

- make

- ./analyzeLightBSM inputfile.txt outFile.root Plastic_BGO

Some general comments:

- find more details about simulation in the readme of the git tag.

- Refer to my thesis and following slides for verifying the results: Chetan_PolStudy_IMPETUS2_30Aug25